CO2 Dragster

Friction and Drag Calculation Activity

- Set-up a ramp using a long board and a table

- Students will measure the length of the ramp (in meters) and record in the excel spreadsheet

- Students will measure the angle of the ramp with a protractor and record in the excel spreadsheet

- Students will measure the mass of their dragster in grams using a triple beam balance or digital scale and record the result in the excel spreadsheet

- Students will sit their dragster at the top of the ramp and release it to roll down the ramp.  Students will collect the time it takes for the car to roll down the ramp.

- Step 5 will be repeated to collect 10 times for the car to roll down the ramp.  The times will be recorded in the excel spreadsheet.  Students should use caution to throw out any times that are unusually high or low compared to the other times. NOTE:  If available, a data collection system such as Vernier should be used to collect the time it takes for the dragster to roll down the ramp.

- If students are having trouble getting their car to roll down the ramp without running off, they will need to modify their car to correct the issues before they are able to collect the times.

- Once students have collected the required information and entered in the spreadsheet, they will be able to determine the amount of force generated by the combination of friction and drag on their car.  Students can then compare their results with other students.

- A copy of the spreadsheet should be placed in the engineering notebook.

- If available, students can also use a wind tunnel to analyze the drag on their CO2 car and compare this information with the other students.