Friction

Name__________________________________________period__________________

- If a boulder has a mass of 50kg, what is its normal force (N)?

- If you push this rock, how much force will it take to keep it moving if the kinetic coefficient of friction (µk) is 0.25?

- Determine the force of friction (Ffriction) between a crate and the ground if the crate weighs 750 N and µ = 0.12.

- What is the coefficient of static friction (µs) if it requires 45N of force to move an 80N crate?

- A 1000g mass is moved by a 2.0N force. What is the coefficient of friction?

- A large piece of granite has a coefficient with the ground of 0.85. 950N of force are required to slide it. What is the mass of the stone?

- A 5.0 kg block has a coefficient of friction of 0.25 and is at rest. If 25N of force is applied, what is the acceleration of the block?

- Find the acceleration of a 20kg object if a 180N force moves it from rest to a speed of 40m/sec in 6 seconds.

- What is the force of friction and the coefficient of friction in problem 8?

- A 150kg load accelerates at a rate of 1.5 m/sec2 . What is the magnitude of the responsible force if the coefficient of friction is 0.18?