Friction

Name__________________________________________period__________________

- If a boulder has a mass of 50kg, what is its normal force (N)?

FN = mg;  FN= 50kg x 9.8 m/sec  =490N

- If you push this rock, how much force will it take to keep it moving if the kinetic coefficient of friction (µk) is 0.25?

FN = mg;  FN= 50kg x 9.8m/sec2 = 490N; F = µ (FN); F = 0.25 x 490N = 122.5N

- Determine the force of friction (Ffriction) between a crate and the ground if the crate weighs 750 N and µ = 0.12.

F = µ (FN); 0.12 x 750N = 90N

- What is the coefficient of static friction (µs) if it requires 45N of force to move an 80N crate?

F = µ(FN); 45N = µ (80) ; 45N/80N = µ = 0.56

- A 1000g mass is moved by a 2.0N force. What is the coefficient of friction?

W = mg; 1.0 kg x 9.8 m/sec = 9.8N

µ = F/FN;  µ = 2.0N/ 9.8N = 0.20

- A large piece of granite has a coefficient with the ground of 0.85. 950N of force are required to slide it. What is the mass of the stone?

F = µ(FN); 950N = 0.85 (FN);  FN = 950N/0.85 = 1118N

m = W/g; m = 1118N/9.8m/sec2 = 114 kg

- A 5.0 kg block has a coefficient of friction of 0.25 and is at rest. If 25N of force is applied, what is the acceleration of the block?

W = mg;  W = 5.0 kg x 9.8 m/sec2 = 49N

F = µ (FN); F = 0.25 x 49N = 12.25N

F = ma; a = F/m; a = 12.25N/5.0kg = 2.45 m/sec2

- Find the acceleration of a 20kg object if a 180N force moves it from rest to a speed of 40m/sec in 6 seconds.

v = vo + at; a = v – vo/t;  a = ( 40 m/sec – 0)/6 sec = 6.7 m/sec2

- What is the force of friction and the coefficient of friction in problem 8?

FN = mg; F = 20 kg x 9.8 m/sec2  = 196N

µ = F/FN; µ = 180N/ 196N = 0.91

- A 150kg load accelerates at a rate of 1.5 m/sec2 . What is the magnitude of the responsible force if the coefficient of friction is 0.18?

FN = mg; FN = 150kg x 9.8 m/sec2 = 1470N

1470N x 0.18 = 265N

F = Fa – Ff;  Fa = F + Ff;  Fa = 1470 + 265N = 1735N