Coefficient of friction lab

Introduction:

The purpose of this lab is to give students an understanding of the basic concept behind frictional coefficients. Using graphical analysis of the relationship between normal force (N) and frictional force

(F friction), students will be able to derive coefficients.

Purpose:

Daily experiences give us a familiarity with frictional forces. Walking across an icy patch or climbing an incline with soft soled shoes gives a different experience of the force of friction.

In this lab those frictional forces will be observed and evaluated by pulling a weighted object along a flat surface. Provided we are using a constant velocity, the applied force will be used to cancel out frictional forces. This is due to the fact that without any acceleration, we can negate force with respect to Newton’s 2nd Law of Motion (F = ma); therefore, applied force should be equal to frictional resistance.

Objectives:

- Measure the force of friction with different surfaces along the horizontal plane.

- Construct a data table and then graph normal force in relation to the force of friction.

- Determine the coefficient of friction between the variance surfaces.

- Extrapolate and interpolate data points.

- Predict force of friction based upon magnitude of normal forces.

Materials:

- Spring scale

- Wooden blocks, 15 x 8 x 10 cm

- Waxed paper, aluminum foil, and 200 grit sandpaper

- Screw with loop

- String

- Weights: 100, 200, and 500g

- String

- Graph paper and a graphing calculator

Lab procedures:

- Screw the loop into the center of one side of the wooden block so that the wooden block has the largest area on lab table.

- Mass the block of wood and record the mass in kilograms.

- Attach the string to the loop on the wood, then attach the other end of the string to spring scale.

- Using the spring scale as the handle, practice pulling the block along the table making sure that reading on the spring scale is consistent.

- Place 200g of weight on the block and pull the block, recording the force measured on the spring scale. Repeat this at least three times, recording the force each time.

- Add 200g more and then repeat step five. Make sure to record the force each time.

- Keep adding 200g and repeating until a total of 1200g have been added to the block.

- Wrap and tape waxed paper onto the block and repeat steps 5 ,6, and 7.

- Remove the waxed paper, add aluminum foil and repeat steps 5,6, and7.

- Remove the foil, add the sandpaper, and again, repeat steps 5, 6, and 7.

Analysis:

- Calculate the value of the normal force. The normal force can be determined by summing the mass of the block and added masses, in kilograms, and multiplying this value by g (9.8 m/sec2 ). Place these values into the table.

- Using the spring scale measurements, record the value of the frictional forces in the second column.

- After all the data points are in the tables, plot the points on graph paper with the normal force, in newtons,  as the independent variable on the x-axis. Plot the force of friction in newtons on the y-axis.

- Use a straight edge and draw a “best fit” line through your points and determine the slope of the line. ( Y= mx + b). b is where the line crosses the y-axis, and m is the change in y divided by the change in x ( rise over run).

- Next enter your data into a graphing calculator to compare the results with your plotted points. Enter the data points and create a scattergram, then use the linear regression function to find best fit line. Record the slope and intercept.

Application:

1. Using extrapolation, determine the frictional force for 2 and 3 kilograms.

2. Using interpolation, find the force of friction for two force values between .4 and 1.5kg.

3.   Make changes to the lab that will give results based on other untested variables. Include any new materials and additional procedures necessary for the lab extension.

Tables

wood

Waxed paper

Foil

Sandpaper